# PART F

# Case 1.

## External pressure coefficients, Cp,e

LR=LpLm=100⟹Lp=100Lm

In the model:

bm=105 mm⟹bp=10.5 m

dm=48 mm⟹dp=4.8 m

hm=37+53-372=45 mm⟹hp=4.5 m

Roof pitch: α=tan-11.62.4=33.69°

| Wall type | Cp,e | Reference |
| --- | --- | --- |
| Windward wall (W) | 0.7 | Table 5.2(A) |
| Leeward wall (L) | -0.5 | Table 5.2(B), θ=0° Roof pitch α=33.69°>25° db=4.810.5=0.4571>0.3 |
| Sidewalls (S) | Horizontal distance from windward edge between 0 and 4.5m. Cp,e=-0.65 | Table 5.2(C) When the horizontal distance from windward edge is between 0 and 4.5m, the distance is considered as 0 to 1h. When the distance is between 4.5m and 4.8m, the distance is between 1h and 2h. |
|  | Horizontal distance from windward edge between 4.5m and 4.8m. Cp,e=-0.5 |  |
| Upwind slope (U) | -0.2230, 0.2863 | Table 5.3(B) Roof pitch α=33.69°>10° hpdp=4.54.8=0.9375 Linear interpolation on α and hd. |
| Downwind slope (D) | -0.6 | Table 5.3(C) Roof pitch α=33.69°>25° bpdp=10.54.8=2.1875<3 |

## Aerodynamic shape factor, Cshp

Only need to consider frictional drag forces and external pressures for enclosed buildings.

[Cl 5.2(a)]

External pressure: Cshp=Cp,eKaKc,eKlKp

Frictional drag forces: Cshp=CfKaKc,e (Not considered, see page 5)

### External pressure

Cp,e already found

Ka can be found by Table 5.4. with Tributary area A<1 m2

|  | Roofs (U, D) and side walls (S) | Windward walls (W) hp<25 m | Leeward walls (L) h<25 m |
| --- | --- | --- | --- |
| Ka | 1.0 | 1.0 | 1.0 |

Kc,e=0.8 (Table 5.5, Design case (a))

#### Local pressure factor Kl for cladding [Cl 5.4.4]

The value of dimension a,

Walls: a=min0.2b,0.2d,h

Roofs:

- hpbp or hpdp≥0.2, a=min0.2b,0.2d

- Both hpbp and hpdp<0.2, a=2h.

Walls:

a=min0.2bp,0.2dp,hp=min0.2×10.5,0.2×4.8,4.5=0.96

a2=0.9216

0.25a2=0.2304

Roofs:

hpbp=4.510.5=0.4286>0.2

a=min0.2bp,0.2dp=0.96

The context only mentioned “tributary area of each fixing is less than 1 m2 in full scale”. We consider the worst-case scenario here, which is A≤0.25a2.

| Design case | Reference case | Location (Proximity to edge) | Kl |
| --- | --- | --- | --- |
| Windward wall (W) | WA1 | Anywhere | 1.5 |
| Downwind corners of roofs with pitch ≥10° | RC2 | < 0.96m from both roof edge and ridge | 3.0 |
| Upwind roof edges | RA1 | < 0.96m | 1.5 |
|  | RA2 | < 0.48m | 2.0 |
| Downwind side of hips and ridges of roofs with pitch ≥10° | RA3 | < 0.96m | 1.5 |
|  | RA4 | < 0.48m | 2.0 |
| Sidewall near windward wall edges | SA1 | < 0.96m | 1.5 |
|  | SA2 | < 0.48m | 2.0 |
| All other areas | - | - | 1.0 |

#### Permeable cladding reduction factor, Kp [Cl 5.4.5]

We have solid cladding, Kp=1.0

### Frictional drag forces [Cl 5.5]

dphp=4.84.5=1.067<4

dpbp=4.810.5=0.4571<4

No need to consider frictional drag forces hence neither dh nor db is greater than 4.

# Case 2.

## External pressure coefficients, Cp,e

dp=10.5 m

bp=4.8 m

hp=4.5 m

Roof pitch: α=tan-11.62.4=33.69°

| Wall type | Cp,e | Reference |
| --- | --- | --- |
| Windward wall (W) | 0.7 | Table 5.2(A) |
| Leeward wall (L) | -0.29 | Table 5.2(B), θ=90° Roof pitch α=33.69°>25° db=10.54.8=2.1875 Linear interpolation: -0.3+2.1875-24-2-0.2--0.3=-0.290625 |
| Sidewalls (S) | Horizontal distance from windward edge between 0 and 4.5m. Cp,e=-0.65 | Table 5.2(C) When the horizontal distance from windward edge is between 0 and 4.5m, the distance is considered as 0 to 1h. When the distance is between 4.5m and 9m, the distance is between 1h and 2h. Between 9m and 10.5m is in the range of 2h to 3h. |
|  | Horizontal distance from windward edge between 4.5m and 9m. Cp,e=-0.5 |  |
|  | Horizontal distance from windward edge between 9m and 10.5m. Cp,e=-0.3 |  |
| Upwind slope (U) | -0.1578, 0.4024 | Table 5.3(B) Roof pitch α=33.69°>10° hpdp=4.510.5=0.4286 Linear interpolation on α and hd. |
| Downwind slope (D) | -0.6 | Table 5.3(C) Roof pitch α=33.69°>25° bpdp=4.810.5=0.4571<3 |

## Aerodynamic shape factor, Cshp

Only need to consider frictional drag forces and external pressures for enclosed buildings.

[Cl 5.2(a)]

External pressure: Cshp=Cp,eKaKc,eKlKp

Frictional drag forces: Cshp=CfKaKc,e (Not considered, see page 7)

### External pressure

Cp,e already found

Ka can be found by Table 5.4. with Tributary area A<1 m2

|  | Roofs (U, D) and side walls (S) | Windward walls (W) hp<25 m | Leeward walls (L) h<25 m |
| --- | --- | --- | --- |
| Ka | 1.0 | 1.0 | 1.0 |

Kc,e=0.8 (Table 5.5, Design case (a))

#### Local pressure factor Kl for cladding [Cl 5.4.4]

Walls:

a=min0.2bp,0.2dp,hp=min0.2×10.5,0.2×4.8,4.5=0.96

a2=0.9216

0.25a2=0.2304

Roofs:

hpbp=4.54.8=0.9375>0.2

a=min0.2bp,0.2dp=0.96

The context only mentioned “tributary area of each fixing is less than 1 m2 in full scale”. We consider the worst-case scenario here, which is A≤0.25a2.

| Design case | Reference case | Location (Proximity to edge) | Kl |
| --- | --- | --- | --- |
| Windward wall (W) | WA1 | Anywhere | 1.5 |
| Downwind corners of roofs with pitch ≥10° | RC2 | < 0.96m from both roof edge and ridge | 3.0 |
| Upwind roof edges | RA1 | < 0.96m | 1.5 |
|  | RA2 | < 0.48m | 2.0 |
| Downwind side of hips and ridges of roofs with pitch ≥10° | RA3 | < 0.96m | 1.5 |
|  | RA4 | < 0.48m | 2.0 |
| Sidewall near windward wall edges | SA1 | < 0.96m | 1.5 |
|  | SA2 | < 0.48m | 2.0 |
| All other areas | - | - | 1.0 |

The building aspect ratio r is calculated as follows:

r=hpminbp,dp=4.5min4.8,10.5=4.54.8=0.9375<1

#### Permeable cladding reduction factor, Kp [Cl 5.4.5]

We have solid cladding, Kp=1.0

### Frictional drag forces [Cl 5.5]

dphp=10.54.5=2.3333<4,  dpbp=10.54.8=2.1875<4

No need to consider frictional drag forces hence neither dh nor db is greater than 4.